DETAILED ESTIMATE

Name of Work :Construction of Village Resource

Centre (VRC) at KVK, South

Tripura, Birchandra Manu.

Estimated Cost : Rs. 3,55,813.00

TIME FOR COMPLETION : 90 (Ninety)) days.

Description of Village Resource Centre (VRC) at KVK, South Tripura, Birchandra Manu

| Name of the Work | : | A civil construction of VRC |
| --- | --- | --- |
| Description of the Work : |  | The Work will be of 648 sqft of built up area. |
| Estimate framed by | : | Er. Tridib Choudhury, a professional Civil Engineer. |

REPORT

History : The proposed building is a Village Resource Centre (VRC) at KVK, South Tripura, Birchandra Manu, The plinth area of the building is 648 sqft. Accordingly the detailed estimate has been framed for an amount of Rs. 3,55,813.00 (Rupees three lac fifty five thousand eight hundred thirteen) only.

Design Scope : Size 36ft x 18 ft and the height of the building will be ten and half from floor to celling. The plinth will be of 2 ft elevated from the ground. The two sides of the building will be of gravel wall and roof will be GCI sheet on steeled tubular structure. The floor will be of Cement finish. The entrance of the hall, there will be a verandah of size 10 ft x 7 ft with R.C.C. roof with 2 rounds pillars and the floor will be of cement finish. Doors are D 1 and on D 2 and window W 1 to S 5 Box Chajja with grill protection. The measurement is given as below against the D 1 and D 2 = 4.5 ft x 7 ft. of wooden door on iron angled frame, W 1 to W 6 = 6 ft x 4 ft. of aluminium framed glass sliding window.

Specification : As recommended by TSR. 20

Estimated Cost : Rs. 3,55,813.00 (Rupees three lac fifty five thousand eight hundred thirteen) only.

Rate : Based on TSR‐2008

Time : 90 (ninety) days

Method : By contract after inviting tender.

Land : Available.

Estimate Based on : TSR‐2008

Estimate prepared By :	Name : Er. Tridib Choudhury

SEAL of the Company

Name of Work

|  | Construction of Village Resource Centre (VRC) at KVK South Tripura, Birchandramanu |  |  |
| --- | --- | --- | --- |
|  |  | GENERAL ABSTRACT |  |
| (a) | S.H. earth work |  | Rs. 14466.00 |
| (b) | S.H. CEMENT CONCRETE WORK |  | Rs. 55826.00 |
| (c) | S.H. REINFORCED cement concrete work |  | Rs. 107784.00 |
| (d) | S.H. BRICK WORK |  | Rs. 46237.00 |
| (e) | S.H. STEEL & WOOD Work |  | Rs. 25152.00 |
| (f) | S.H. Floring Work |  | Rs. 11035.00 |
| (g) | S.H. FINISHING WORK |  | Rs. 26632.00 |
| (h) | S.H. ROOFING WORK |  | Rs.68681.00 |
|  |  | Total | Rs. 355813.00 |

|  |  | DETAILED ESTIMATE |  |  |
| --- | --- | --- | --- | --- |
| Name of work | Construction of Village Resource Centre(VRC) at KVK South |  |  |  |
|  | Tripura, Birchandramanu |  |  |  |

| Item | TSR Code |  | Particulars of Items |  |  |  | Unit | Quantity | Rate | Amount |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sub Head‐Earth Work |  |  |  |  |  |  |  |
| 1 | TSR‐2008 |  | Earthwork in excavation in all kinds of soil in |  |  |  |  |  |  |  |
|  | Chapter‐2 |  | foundation, trenches of drains (not |  |  |  |  |  |  |  |
|  | Item‐2.3 |  | exceeding 1.5 m in width of 10 sqm. On |  |  |  |  |  |  |  |
|  |  |  | plan) including dressing of sides and |  |  |  |  |  |  |  |
|  |  |  | ramming of bottoms, lift up to 1.5 m, |  |  |  |  |  |  |  |
|  |  |  | including getting out the excavated soil, |  |  |  |  |  |  |  |
|  |  |  | stacking the suitable soil property for using |  |  |  |  |  |  |  |
|  |  |  | the same in the building below floor and |  |  |  |  |  |  |  |
|  |  |  | disposal of surplus excavated soil as |  |  |  |  |  |  |  |
|  |  |  | directed, within a lead of 50 m as directed as |  |  |  |  |  |  |  |
|  |  |  | per clauses of Chapter – 2 of CPWD |  |  |  |  |  |  |  |
|  |  |  | specification |  |  |  |  |  |  |  |
|  |  |  | a. | Without dewatering |  |  |  |  |  |  |
|  |  |  | Coll footing = 1x16x1.00x1.00x0.90 =14.40 |  |  |  |  |  |  |  |
|  |  |  | cum |  |  |  |  |  |  |  |
|  |  |  | Wall footing= 1x1.36.8x0.60x0.60=13.25 |  |  |  |  |  |  |  |
|  |  |  | cum |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | cum | 27.65 | 1.1.5 | 2806.00 |
|  |  |  |  | Total = 27.65 cum |  |  |  |  |  |  |
| 2 | TSR‐2008 |  | Filling in trenches, plinth, under floor, sides |  |  |  |  |  |  |  |
|  | Chapter‐2 |  | of foundations etc. with available excavated |  |  |  |  |  |  |  |
|  | Item‐2.4 |  | suitable soil in regular horizontal layers, |  |  |  |  |  |  |  |
|  |  |  | each not exceeding 200 mm in depth with |  |  |  |  |  |  |  |
|  |  |  | all lead and lifts, consolidating each layer by |  |  |  |  |  |  |  |
|  |  |  | watering and ramming with steel rammers |  |  |  |  |  |  |  |
|  |  |  | of ½ tone roller as per clauses of Chapter – 2 |  |  |  |  |  |  |  |
|  |  |  | of |  |  |  |  |  |  |  |
|  |  |  | Under Floor = 1x14.75x7.5x0.50=55.31 cum |  |  |  |  |  |  |  |
|  |  |  | Side of footing=2/3x27.65 |  | =18.43cum |  |  |  |  |  |
|  |  |  |  |  |  |  | Cum | 73.74 | 35.5 | 26.18.00 |
|  |  |  |  | Total | 73.74 cum |  |  |  |  |  |
| 3 | TSR‐2008 |  | Filling in trenches, plinth, under floor, |  |  |  |  |  |  |  |
|  | Chapter‐2 |  | foundations etc. as per design with dands |  |  |  |  |  |  |  |
|  | Item‐2.8 |  | with all lifts, transporting to site, spreading |  |  |  |  |  |  |  |
|  |  |  | in regular horizontal layers, watering, |  |  |  |  |  |  |  |
|  |  |  | grading to required slope and compacting |  |  |  |  |  |  |  |
|  |  |  | each layer by using plate compactor or by |  |  |  |  |  |  |  |
|  |  |  | any suitable method as per clauses of |  |  |  |  |  |  |  |
|  |  |  | Chaper – 2 of CPWD specification. |  |  |  |  |  |  |  |
|  |  |  | 1. | With fine sand obtained from local |  |  |  |  |  |  |
|  |  |  |  | quarry. |  |  |  |  |  |  |
|  |  |  | Cool footing=1x16x1.00x1.00x0.075=1.20 |  |  |  |  |  |  |  |
|  |  |  | cum |  |  |  |  |  |  |  |
|  |  |  | Wall footing=1x1x52.8x0.60x0.075=2.38 |  |  |  |  |  |  |  |
|  |  |  | cum |  |  |  |  |  |  |  |

|  |  |  | Under Floor=1.15.00x7.75x0.15 = 17.44 cum |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1x3.00x2.15x0.15 = 0.97 cum |  |  |  |  |  |  |
|  |  |  | Total = 21.99 cum |  |  | Cum | 21.99 | 411.2 | 9042.00 |
|  |  |  | Sub Head‐Cement Concrete Work |  |  |  |  |  |  |
| 4 | TSR‐2008 |  | Providing and laying in position cement |  |  |  |  |  |  |
|  | Chapter‐3 |  | concrete of specified grade including the |  |  |  |  |  |  |
|  | Item‐3.1 |  | cost of Form work, all work upto plinth level |  |  |  |  |  |  |
|  |  |  | as per clauses of Chapter‐4 of CPWD |  |  |  |  |  |  |
|  |  |  | specification |  |  |  |  |  |  |
|  |  |  | 1. 1:4:8 (1 cement :4 fine sand:8graded |  |  |  |  |  |  |
|  |  |  | well burnt brick aggregate of 40 mm |  |  |  |  |  |  |
|  |  |  | nominal size) |  |  |  |  |  |  |
|  |  |  | Cool footing=1x16x1.00x1.00x0.075=1.20 |  |  |  |  |  |  |
|  |  |  | cum |  |  |  |  |  |  |
|  |  |  | Wall footing=1x1x52.8x0.60x0.075 = 2.38 |  |  |  |  |  |  |
|  |  |  | cum |  |  |  |  |  |  |
|  |  |  | Under Floor=1x15.00x7.75x0.10 = 11.63 |  |  |  |  |  |  |
|  |  |  | cum |  |  |  |  |  |  |
|  |  |  | 1x3.00x2.15x0.10 | = 0.27 cum |  |  |  |  |  |
|  |  |  |  |  |  | Cum | 16.13 | 3461 | 55826.00 |
|  |  |  | Total = | 16.13 cum |  |  |  |  |  |
|  |  |  | Sub Head – R.C.C. Work |  |  |  |  |  |  |
| 5 | TSR‐2008 |  | Providing and laying in position reinforced |  |  |  |  |  |  |
|  | Chapter‐4 |  | cement concrete of specified grade |  |  |  |  |  |  |
|  | Item‐4.1 |  | including the cost of Form work. But |  |  |  |  |  |  |
|  |  |  | excluding finishing and reinforcement, all |  |  |  |  |  |  |
|  |  |  | work upto plinth level as per |  |  |  |  |  |  |
|  |  |  | 1 1:1.5 : 3 (1 cement : 1.5 fine sand : 3 |  |  |  |  |  |  |
|  |  |  | graded stone |  |  |  |  |  |  |
|  |  |  | Aggregate of 20 mm nominal size) |  |  |  |  |  |  |
|  |  |  | Coll footing=1x16x1.00x1.00x0.15=2.40 cum |  |  |  |  |  |  |
|  |  |  | 1x16x15/4( ^2+.25^2+2(1x25)=0.94 cum |  |  |  |  |  |  |
|  |  |  | Total = | 3.34 cum |  | Cum | 3.34 | 7397 | 24707.00 |
| 6 | TSR‐2008 |  | Providing and laying in positions reinforced |  |  |  |  |  |  |
|  | Chapter‐4 |  | cement concrete of specified grade in |  |  |  |  |  |  |
|  | Item‐4.3 |  | suspended floors, roofs hasving slope upto |  |  |  |  |  |  |
|  |  |  | `15 degree, landings, balconies, shelves, |  |  |  |  |  |  |
|  |  |  | chajjas etc. upto floor five level including the |  |  |  |  |  |  |
|  |  |  | cost of frame work, but excluding finishing |  |  |  |  |  |  |
|  |  |  | and |  |  |  |  |  |  |
|  |  |  | 1. 1 : 1.5 : 3 (1 cement : 1.5 fine sand : |  |  |  |  |  |  |
|  |  |  | 3 graded stone aggregate of 20 mm |  |  |  |  |  |  |
|  |  |  | nominal |  |  |  |  |  |  |

|  |  |  | Ent. Slab = 1.3.00x2.15x0.11=0.71 cum |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Chajja | = 1x6x1.9x0.50x0.10=0.57 cum |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Cum | 1.28 | 8215 | 10515,00 |
|  |  |  |  | Total | = | 1.28 cum |  |  |  |  |  |
| 7 | TSR‐2008 |  | Providing and laying in position reinforced |  |  |  |  |  |  |  |  |
|  | Chapter‐4 |  | cement concrete of specified grade in |  |  |  |  |  |  |  |  |
|  | Item no. |  | beams, lintels, bands, plain window sills, |  |  |  |  |  |  |  |  |
|  | 4.4 |  | stair cases, spiral stair cases excluding pre‐ |  |  |  |  |  |  |  |  |
|  |  |  | cast spiral stair case etc. upto floor five level |  |  |  |  |  |  |  |  |
|  |  |  | including the cost of frame work, but |  |  |  |  |  |  |  |  |
|  |  |  | excluding finishing and reinforcement, as |  |  |  |  |  |  |  |  |
|  |  |  | per clauses of Chapter‐ 5 of |  |  |  |  |  |  |  |  |
|  |  |  | 1. | 1 : 1.5 : 3 (1 cement : 1.5 fine sand |  |  |  |  |  |  |  |
|  |  |  |  | : 3 graded stone aggregate of 20 |  |  |  |  |  |  |  |
|  |  |  |  | mm nominal |  |  |  |  |  |  |  |
|  |  |  | Plinth beam=1x52.80x0.25x0.25=3.30 cum |  |  |  |  |  |  |  |  |
|  |  |  | Lintel beam=1x45.5x0.125x0.15 |  |  |  |  |  |  |  |  |
|  |  |  | Entrance beam = 1x7.20x0.25x0.10=0.34 |  |  |  |  |  |  |  |  |
|  |  |  | cum |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Cum | 4.49 | 8581 | 38527.00 |
|  |  |  |  | Total |  | 4.49 cum |  |  |  |  |  |
| 8 | TSR‐2008 |  | Providing and laying in position reinformed |  |  |  |  |  |  |  |  |
|  | Chapter‐4 |  | cement concrete of specified grade |  |  |  |  |  |  |  |  |
|  | Item no. |  | including the cost of Form work, but |  |  |  |  |  |  |  |  |
|  | 4.2 |  | excluding finishing and reinforcement, upto |  |  |  |  |  |  |  |  |
|  |  |  | floor five level as per clauses of |  |  |  |  |  |  |  |  |
|  |  |  | a. | In plinth & skirting courses, fillets, |  |  |  |  |  |  |  |
|  |  |  |  | columns, pillars, post & struts etc. |  |  |  |  |  |  |  |
|  |  |  | Column = 1x14x0.25x0.25x4.4=3.85 cum |  |  |  |  |  |  |  |  |
|  |  |  |  | 1x2x3.14/4x25^2x3.3=0.32 cum |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Cum | 4.17 | 8162 | 34035.00 |
|  |  |  |  | Total | = 4.17 cum |  |  |  |  |  |  |
|  |  |  | Sub Head‐Brick Work |  |  |  |  |  |  |  |  |
| 9 | TSR‐2008 |  | First class brick work in foundation and |  |  |  |  |  |  |  |  |
|  | Chapter‐6 |  | plinth including cost of all materials required |  |  |  |  |  |  |  |  |
|  | Item no. |  | and as per clauses of chapter‐6 of CPWD |  |  |  |  |  |  |  |  |
|  | 6.2 |  | 1. | In Cement mortar 1:6 ( 1 cement : |  |  |  |  |  |  |  |
|  |  |  |  | 6 fine sand) |  |  |  |  |  |  |  |
|  |  |  | Wall footing = 1x52.8x0.254x0.40=5.36 cum |  |  |  |  |  |  |  |  |
|  |  |  |  | Total = | 5.36 cum |  |  | Cum | 5.36 | 2835 | 15195 |
| 10 | TSR‐2008 |  | Half brick masonry with first class bricks in |  |  |  |  |  |  |  |  |
|  | Chapter‐6 |  | superstructure above plinth level upto floor |  |  |  |  |  |  |  |  |
|  | Item 6.13 |  | five level including cost of all materials |  |  |  |  |  |  |  |  |
|  |  |  | required and as per clauses of chapter‐6 of |  |  |  |  |  |  |  |  |
|  |  |  | CPWD specification. |  |  |  |  |  |  |  |  |
|  |  |  | 1 In Cement mortar 1:4 ( 1 cement : 4 |  |  |  |  |  |  |  |  |
|  |  |  |  | fine sand) |  |  |  |  |  |  |  |
|  |  |  | Wall = 1x30.00x3.00=90.00 sqm |  |  |  |  |  |  |  |  |

|  |  |  |  | Less = D=1x2x1.4x2.1 = 5.88 sqm |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | W=1x6x1.80x1.20 = 12.96 sqm |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Sqm | 71.16 | 436.2 | 31042.00 |
|  |  |  |  |  | Total = 71.16 sqm |  |  |  |  |  |  |
| 11 | TSR‐2008 |  | Providing and fixing glazing in aluminium |  |  |  |  |  |  |  |  |
|  | Chapter‐ |  | aluminium doors, windows, ventilator, |  |  |  |  |  |  |  |  |
|  | 10 |  | shutters and partition frames with P |  |  |  |  |  |  |  |  |
|  | Item no. |  | VC/neoprene gasket etc. complete as per |  |  |  |  |  |  |  |  |
|  | 10.3 |  | architectutal drawings and the direction of |  |  |  |  |  |  |  |  |
|  |  |  | Engineer‐In‐charge and as per clauses of |  |  |  |  |  |  |  |  |
|  |  |  | chapter 24 of CPWD specification |  |  |  |  |  |  |  |  |
|  |  |  |  | A. With sglass panes of 4.0 m |  |  |  |  |  |  |  |
|  |  |  |  |  | thickness (weight not less than 10.0 |  |  |  |  |  |  |
|  |  |  |  |  | kg/sqm) |  |  |  |  |  |  |
|  |  |  |  |  | W=1x6x1.80x1.20=12.96 sqm |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Sqm | 12.96 | 520.6 | 6747.00 |
|  |  |  |  |  | Total = | 12.96 sqm |  |  |  |  |  |
| 12 | TSR‐2008 |  | Providing and fixing M.S. grills of required |  |  |  |  |  |  |  |  |
|  | Chapter‐9 |  | pattern in frames of windows etc. with M.S. |  |  |  |  |  |  |  |  |
|  | Item no. |  | flat, solid square of solid round bars etc. all |  |  |  |  |  |  |  |  |
|  | 8.19 |  | complete as per clauses of Chapter‐9 of |  |  |  |  |  |  |  |  |
|  |  |  | CPWD specification |  |  |  |  |  |  |  |  |
|  |  |  |  | 1. Fixed to openings in walls with rawf |  |  |  |  |  |  |  |
|  |  |  |  |  | plugs, screws etc. |  |  |  |  |  |  |
|  |  |  | W=1x6x1.80x1.20=12.96 sqm @ 10 |  |  |  |  |  |  |  |  |
|  |  |  | kg/sqm=129.60 kg |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Kg | 129.6 | 69.34 | 8986.0 |
|  |  |  |  |  | Toal = 129.6 kg |  |  |  |  |  |  |
| 13 | TSR‐2008 |  | Providing and fixing T‐iron frames for doors, |  |  |  |  |  |  |  |  |
|  | Chapter‐9 |  | windows and ventilators of mild steel tea |  |  |  |  |  |  |  |  |
|  | Item no. |  | sections, joints mitred and welded with |  |  |  |  |  |  |  |  |
|  | 9.11 |  | (15x3mm)lugs 10 cm long embedded in |  |  |  |  |  |  |  |  |
|  |  |  | cement concrete blocks(15x10x10 cm) of |  |  |  |  |  |  |  |  |
|  |  |  | 1:3:6 ( 1 cement :3 fine sand:3 graded brick |  |  |  |  |  |  |  |  |
|  |  |  | aggregate 20 mm nominal size) or with |  |  |  |  |  |  |  |  |
|  |  |  | wooden plugs and screws or with fixing clips |  |  |  |  |  |  |  |  |
|  |  |  | or with bolts and nuts as required including |  |  |  |  |  |  |  |  |
|  |  |  | fixing of necessary butt hinges and screws |  |  |  |  |  |  |  |  |
|  |  |  | amnd applying a priming cost of approved |  |  |  |  |  |  |  |  |
|  |  |  | steel primer as per clauses of chapter 10 of |  |  |  |  |  |  |  |  |
|  |  |  | CPWD specification. |  |  |  |  |  |  |  |  |
|  |  |  |  | D = 1x2x2x2.1=8.4 m |  |  |  |  |  |  |  |
|  |  |  |  |  | 1x2x1x1.4 = 2.8 m |  |  |  |  |  |  |
|  |  |  | 11.20m @ 3.5 Kg/m =39.20 kg |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Kg | 39.2 | 77.34 | 3032.00 |
|  |  |  |  | Total = 39.2 kg |  |  |  |  |  |  |  |
| 14 | TSR‐2008 |  | Providing and fixing paneled/or paneled and |  |  |  |  |  |  |  |  |

|  | Chapter‐8 |  | glazed shutters for doors, windows and |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Item no. |  | clerestory windows including bright finished |  |  |  |  |  |  |  |
|  | 8.5 |  | black enameled M.S. butt hinges with |  |  |  |  |  |  |  |
|  |  |  | necessary screws (excluding paneling and or |  |  |  |  |  |  |  |
|  |  |  | glazing.which will be paid for separately). |  |  |  |  |  |  |  |
|  |  |  | But including the wooden beading of 20 x 12 |  |  |  |  |  |  |  |
|  |  |  | mm for |  |  |  |  |  |  |  |
|  |  |  | B Double leaf door shutter |  |  |  |  |  |  |  |
|  |  |  | 1. | Gamair wood |  |  |  |  |  |  |
|  |  |  |  | a. 35 mm thick shutters |  |  |  |  |  |  |
|  |  |  | D=1x2x1.40x2.10=5.88 sqm |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Sqm | 5.88 | 936.7 | 5508.00 |
|  |  |  |  | Total = | 5.88 sqm |  |  |  |  |  |
| 15 | TSR‐2008 |  | Providing and fixing ISI marked aluminium |  |  |  |  |  |  |  |
|  | Chapter‐8 |  | sliding door bolts anodished (anodized |  |  |  |  |  |  |  |
|  | Item no., |  | coating not less than grade AC 10 as per IS |  |  |  |  |  |  |  |
|  | 8.41 |  | 1868) transparent or dyed to required |  |  |  |  |  |  |  |
|  |  |  | colour or shade with necessary screws etc. |  |  |  |  |  |  |  |
|  |  |  | complete as per clauses of Chapter – 9 of |  |  |  |  |  |  |  |
|  |  |  | CPWD specification |  |  |  |  |  |  |  |
|  |  |  | 1. 300 x 16 mm |  |  |  |  |  |  |  |
|  |  |  | Total = 2 nos |  |  |  | Nos | 2 | 218.3 | 437.00 |
| 16 | TSR‐2008 |  | Providing and fixing ISI marked aluminium |  |  |  |  |  |  |  |
|  | Chapter‐8 |  | tower bolts anodized (anodished coating not |  |  |  |  |  |  |  |
|  | Item no. |  | less than grade AC 10 as per IS 1868) |  |  |  |  |  |  |  |
|  | 8.42 |  | stransparent or dyed to required colour or |  |  |  |  |  |  |  |
|  |  |  | shade with necessary screws etc. complete |  |  |  |  |  |  |  |
|  |  |  | as per clauses of Chapter – 9 of CPWD |  |  |  |  |  |  |  |
|  |  |  | specification |  |  |  |  |  |  |  |
|  |  |  | 1. | 300x10 mm |  |  |  |  |  |  |
|  |  |  |  | Total = 2 nos |  |  | Nos | 2 | 84.1 | 168.00 |
| 17 | TSR‐2008 |  | Providing and fixing ISI marked aluminium |  |  |  |  |  |  |  |
|  | Chapter‐8 |  | handles with plate anodized (anodized |  |  |  |  |  |  |  |
|  | Item no. |  | coating not less than grade AC 10 as per IS |  |  |  |  |  |  |  |
|  | 8.45 |  | 1868) transparent or dyed to required |  |  |  |  |  |  |  |
|  |  |  | colour or shade with necessary screws etc. |  |  |  |  |  |  |  |
|  |  |  | complete as per clauses of Chapter – 9 of |  |  |  |  |  |  |  |
|  |  |  | CPWD specification. |  |  |  |  |  |  |  |
|  |  |  | 1. | 150 mm |  |  |  |  |  |  |
|  |  |  |  | Total = 4 Nos |  |  | Nos | 4 | 68.5 | 274.00 |
|  |  |  | Sub Head‐Flooring Work |  |  |  |  |  |  |  |
| 18 | TSR‐2008 |  | Providing and laying cement concrete 1:2:4 |  |  |  |  |  |  |  |
|  | Chapter‐ |  | (1 cement : 2 fine sand :4 graded well burnt |  |  |  |  |  |  |  |
|  | 11 |  | brick aggregate) flooring finished with a |  |  |  |  |  |  |  |

|  | Item no. |  | floating cost of neat cement including |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11.9 |  | cement slurry, rounding of edges and strips |  |  |  |  |  |  |  |  |  |
|  |  |  | etc. complete as per clauses of Chapter – 11 |  |  |  |  |  |  |  |  |  |
|  |  |  | of CPWD specification. |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1. 25 mm thick with 12.5 mm stone |  |  |  |  |  |  |  |  |
|  |  |  |  |  | aggregate |  |  |  |  |  |  |  |
|  |  |  |  |  | Floor = 1x11.00x5.50=60.50 sqm |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Sqm | 60.5 | 182.4 | 11035.00 |
|  |  |  |  |  |  | Total | 60.5 sqm |  |  |  |  |  |
|  |  |  | Sub Head‐Finishing Work |  |  |  |  |  |  |  |  |  |
| 19 | TSR‐2008 |  | 12 mm cement plaster in single layer |  |  |  |  |  |  |  |  |  |
|  | Chapter‐ |  | including cost of materials required and |  |  |  |  |  |  |  |  |  |
|  | 13 |  | finishing even and smooth and curing |  |  |  |  |  |  |  |  |  |
|  | Item no. |  | complete as per clauses of chapter 13 of |  |  |  |  |  |  |  |  |  |
|  | 13.1 |  | CPWD specification. |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1. Cement mortar 1:6 (1 cement :6 fine |  |  |  |  |  |  |  |  |
|  |  |  |  |  | sand) |  |  |  |  |  |  |  |
|  |  |  |  | 1x4x10.00x3.00=120.00 |  |  |  |  |  |  |  |  |
|  |  |  |  | 1x4x6.00x3.00 = 72.00 |  |  |  |  |  |  |  |  |
|  |  |  |  | Less D=1x4x(1/2)x1.4x2.1 =5.88 |  |  |  |  |  |  |  |  |
|  |  |  |  | W=1x12x(1/2)x1.8x1.2=5.88 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Sqm | 173.16 | 71.63 | 12403.00 |
|  |  |  |  | Total |  | 173.16 sqm |  |  |  |  |  |  |
| 20 | TSR‐2008 |  | Wall painting with plastic emulsion paint |  |  |  |  |  |  |  |  |  |
|  | Chapter‐ |  | (two or more coats) of required colour of |  |  |  |  |  |  |  |  |  |
|  | 13 |  | approved brand and manufacture on new |  |  |  |  |  |  |  |  |  |
|  | Item no. |  | work to give an even shade including cost of |  |  |  |  |  |  |  |  |  |
|  | 13.4 |  | materials required and as per clauses of |  |  |  |  |  |  |  |  |  |
|  |  |  | chapter 13 of CPWD specification. |  |  |  |  |  |  |  |  |  |
|  |  |  | Inside wall =1x2x15.00x3.20=96.00 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 1x2x7.75x3.20=49.60 |  |  |  |  |  |  |
|  |  |  | Less‐E=1x2x(1/2x1.4x2.1=2.94 |  |  |  |  |  |  |  |  |  |
|  |  |  | W=1x6x(1/2)x1.8x1.2=6.48 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Sqm | 136.18 | 44.12 | 6008.00 |
|  |  |  |  | Total = |  | 136.18 sqm |  |  |  |  |  |  |
| 21 | TSR‐2008 |  | Finishing walls with acrylic smooth exterior |  |  |  |  |  |  |  |  |  |
|  | Chapter‐ |  | paint of required shade of approved brand |  |  |  |  |  |  |  |  |  |
|  | 13 |  | and manufacture on new work (two or more |  |  |  |  |  |  |  |  |  |
|  | Item no. |  | coats applied @1.667 kg/10 sqm) over and |  |  |  |  |  |  |  |  |  |
|  | 13.2 |  | including base coat water proofing cement |  |  |  |  |  |  |  |  |  |
|  |  |  | paint applied @ 2.20 kg/10 sqm complete |  |  |  |  |  |  |  |  |  |
|  |  |  | including cost of materials required and as |  |  |  |  |  |  |  |  |  |
|  |  |  | per clauses of chapter |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Outside wall = 1x2x15.00x3.20=96.00 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 1x2x7.75x3.20 = 49.60 |  |  |  |  |  |  |
|  |  |  |  | Less‐D=1x2x(1/2)x1.4x2.1 = 2.94 |  |  |  |  |  |  |  |  |

|  |  |  | W=1x6x(1/2)x1.8x1.2=6.48 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Total = 136.18 sqm |  | Sqm | 136.18 | 60.37 | 8221.00 |
| 22 | TSR‐2008 |  | Providing and fixing steel work in built up |  |  |  |  |  |  |
|  | Chapter= |  | tubular trusses including cutting. Hoisting, |  |  |  |  |  |  |
|  | 9 |  | fixing in position and applying a priming coat |  |  |  |  |  |  |
|  | Item no. |  | of approved steel primer, welded and bolted |  |  |  |  |  |  |
|  | 9.12 |  | including special shaped washers etc. |  |  |  |  |  |  |
|  |  |  | complete as per clauses of chapter‐10 of |  |  |  |  |  |  |
|  |  |  | CPWD specification. |  |  |  |  |  |  |
|  |  |  | 1. With mild steel tubes Hot finished |  |  |  |  |  |  |
|  |  |  |  | welded (HFW)type medium class. |  |  |  |  |  |
|  |  |  |  | Total = 350 kg |  | Kg | 350 | 79.94 | 27979.00 |
| 23 | TSR‐1008 |  | Providing and fixing corrugated G.S. Sheet |  |  |  |  |  |  |
|  | Chapter‐ |  | roofing fixed with polymer coated J or L |  |  |  |  |  |  |
|  | 12 |  | hooks 8 mm diameter, bolts and nuts 6 mm |  |  |  |  |  |  |
|  | Item no. |  | diameter with bitumen and G.I. limpet |  |  |  |  |  |  |
|  | 12.1 |  | washers or with G.I. limpet washers filled |  |  |  |  |  |  |
|  |  |  | with lead and including a coat of approved |  |  |  |  |  |  |
|  |  |  | steel primer and two coats of approved |  |  |  |  |  |  |
|  |  |  | epoxy paint on overlapping of sheets |  |  |  |  |  |  |
|  |  |  | complete (upto a pitch of 60 degrees) over a |  |  |  |  |  |  |
|  |  |  | built structure including all types of |  |  |  |  |  |  |
|  |  |  | necessary scaffoldings etc. but excluding the |  |  |  |  |  |  |
|  |  |  | cost of the purlins, rafters and trusses etc. as |  |  |  |  |  |  |
|  |  |  | per clauses of Chyapter‐12 of CPWD |  |  |  |  |  |  |
|  |  |  | specification |  |  |  |  |  |  |
|  |  |  | 1. | 0.63 mm thick with zinc coating not |  |  |  |  |  |
|  |  |  |  | less than 275 gm/m2 |  |  |  |  |  |
|  |  |  |  | roof=1x2x11.00x2.75=60.50 sqm |  |  |  |  |  |
|  |  |  |  |  |  | Sqm | 60.5 | 599.4 | 36266.00 |
|  |  |  |  | Total = 60.5 sqm |  |  |  |  |  |
| 24 | TSR‐2008 |  | Providing and fixing ridges or hips of 60 cm |  |  |  |  |  |  |
|  | Chapter‐ |  | overall width of plain G.S. Sheet (class‐1) |  |  |  |  |  |  |
|  | 12 |  | fixed with polymer coated J or L hooks 8 mm |  |  |  |  |  |  |
|  | Item no. |  | diameter, bolts and nuts 6 mm diameter |  |  |  |  |  |  |
|  | 12.4 |  | with bitumen and G.I. limpet washers and |  |  |  |  |  |  |
|  |  |  | including a coat of approved steel primer |  |  |  |  |  |  |
|  |  |  | and two coats of approved epoxy paint on |  |  |  |  |  |  |
|  |  |  | overlapping of sheets complete as per |  |  |  |  |  |  |
|  |  |  | clauses of Chapter – 12 of CPWD |  |  |  |  |  |  |
|  |  |  | specification. |  |  |  |  |  |  |
|  |  |  | 1. | 0.63 mm thick with zinc coating not |  |  |  |  |  |

|  |  |  | less than 275 gm/m2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | roof = 1x1x11.00 = 11.00 sqm |  |  |  |  |  |
|  |  |  |  |  | Mtr. | 11 | 403.3 | 4436.00 |
|  |  |  | Total = 11.00 mtr. |  |  |  |  |  |
|  |  |  |  |  |  | Total | Rs. 355813.00 |  |

Er. Tridib Choudhury

B.E. CIVIL

Professional Civil Engineer